Evaluation 1: Baseline and Problem Understanding

Measuring the Systems Cost of Heterogeneous KV Quantization

Shivam Patel (2025202030)  ·  Suyash Pande (2025201063)  ·  Divyanshu Jain (2025201059)

CS3.406 Programming AI Accelerators  ·  Plan as of 17 October 2026

# 1. Goal of Evaluation 1

Evaluation 1 (week 5, 20% of the grade) presents baseline results and the plan they lead to. We do not build the custom kernel yet. We answer two questions with measurements on Qwen2.5-7B-Instruct:

- Which KV heads matter, and does it depend on the task? We profile per-head attention behaviour and per-head quantization sensitivity across three tasks.

- Does KV quantization reduce or increase the cost of inference? We measure throughput, latency and memory before and after uniform KV quantization, both in systems that dequantize naively and in systems with fused kernels.

The project question is unchanged from the proposal: is per-head mixed-precision KV free at inference time, or do dispatch, load imbalance and unpacking costs eat the savings? Evaluation 1 gives us the sensitivity ranking that sets the bit-widths, and the cost baseline that any mixed-precision kernel must beat.

The course rubric asks Evaluation 1 to show a working baseline, reliable measurements, initial profiling, an identified bottleneck and a proposed optimization strategy. Its key question is: what exactly are we trying to improve, and how do we know it is the bottleneck?

# 2. Hardware and Software

The college now gives us remote access to an NVIDIA RTX PRO 5000 (Blackwell, 48 GB). This replaces the 24 GB target in the proposal, so we can run a full 7–8B model at long context and drop the 3B model.

| Item | Value |
| --- | --- |
| GPU | NVIDIA RTX PRO 5000, Blackwell architecture (sm_120) |
| Memory | 48 GB |
| Memory bandwidth | About 1.3 TB/s (approximate; we will measure it ourselves) |
| CUDA | 12.8 or later (required for Blackwell) |
| PyTorch | 2.7 or later, cu128 build |
| Profilers | Nsight Systems, Nsight Compute, nvidia-smi, NVML |
| Libraries | HuggingFace transformers, optimum-quanto, vLLM; LMDeploy if a Blackwell build exists |

Week 1 checks:

- Install the cu128 PyTorch build. Older wheels fail on sm_120 with "no kernel image available".

- Confirm ncu can read performance counters (no ERR_NVGPUCTRPERM).

- Ask whether we can lock clocks (nvidia-smi -lgc) and get exclusive access during measurement runs.

- Confirm vLLM (and LMDeploy) run on this GPU before relying on them as baselines.

- Measure real peak DRAM bandwidth with a copy kernel, so roofline numbers use our own figure.

# 3. Model

We use Qwen2.5-7B-Instruct for Evaluation 1. Access to Llama-3.1-8B-Instruct is pending; when it arrives, Llama becomes the second model family, and comparing the two is itself an analysis point.

| Property | Qwen2.5-7B-Instruct | Llama-3.1-8B-Instruct (later) |
| --- | --- | --- |
| Layers | 28 | 32 |
| Query heads per layer | 28 | 32 |
| KV heads per layer | 4 | 8 |
| Query heads sharing one KV head | 7 | 4 |
| Head dimension | 128 | 128 |
| Weights in BF16 | About 15 GB | About 16 GB |
| KV cache per token (FP16) | 56 KB | 128 KB |
| KV cache at 32K context | 1.75 GB | 4 GB |
| Native context | 32K (128K with YaRN) | 128K |
| KV heads to rank (quantization units) | 28 × 4 = 112 | 32 × 8 = 256 |

KV cache per token = 2 (K and V) × layers × KV heads × head dim × 2 bytes. For Qwen: 2 × 28 × 4 × 128 × 2 = 57,344 bytes = 56 KB.

Why Qwen works well here:

- It fits easily: 15 GB of weights plus 1.75 GB of KV per 32K sequence leaves room for batch 8 at 32K (about 29 GB total) on 48 GB.

- Its keys contain large outlier channels, so it is known to be more sensitive to KV quantization than Llama. Differences between heads should be easy to see.

- It needs no access approval.

What a "head" means for us. With grouped-query attention, 7 query heads share each KV head. The KV cache stores KV heads, so quantization happens per KV head. We therefore work at two granularities:

- Attention behaviour is recorded per query head (28 × 28 = 784 heads), because that is where attention patterns live.

- Sensitivity and bit-width are assigned per KV head (112 units), because that is what can be stored at a different precision.

# 4. Tasks and Datasets

We use three tasks because head importance depends on the task. Published work shows that a small set of "retrieval heads" does almost all long-context copying (Retrieval Head, Wu et al., 2024), and that heads split into retrieval and streaming types (DuoAttention, 2024). A plain-text calibration set would rank those heads as unimportant; quantizing them hard would keep perplexity fine but break retrieval.

| Task | Dataset | Context | Heads expected to matter | Quality metric |
| --- | --- | --- | --- | --- |
| Language modeling | WikiText-2, PG19 | 4K, 8K, 16K, 32K | Local and general heads | Perplexity |
| Long-context retrieval | Needle-in-a-haystack / passkey (we generate it) | 4K–32K, several needle depths | Retrieval heads (few, critical) | Retrieval accuracy |
| Multi-step reasoning | GSM8K with chain-of-thought | Short prompt, long generation | Heads used during decode; errors build up over many tokens | Exact-match accuracy |
| Long-document QA (optional) | LongBench subset (HotpotQA or code completion) | 8K–16K | Multi-hop and copying heads | F1 / edit similarity |

Rules to keep the results honest:

- Calibrate on train splits, evaluate on test splits. Head rankings come from PG19 train, our own generated needles and GSM8K train. Results are reported on WikiText-2 test, PG19 test, new needle depths and lengths, and GSM8K test.

- Compare at equal cost. Quality is plotted against average bits per KV value, including scale and zero-point overhead, not against scheme names.

- Score each task separately before combining, so we can show whether the rankings agree.

# 5. Analysis 1: Which Heads Activate

We run each task through the FP16 model and record how each query head distributes its attention. This shows what each head does; Section 6 tests whether that matters for quantization.

| Statistic | Definition | What it identifies |
| --- | --- | --- |
| Attention entropy | Entropy of each head's attention distribution, averaged over query positions | Focused heads (low) vs. diffuse heads (high) |
| Sink fraction | Share of attention placed on the first token | "Attention sink" heads that do little real work |
| Local fraction | Share of attention within the last 64 tokens | Local / streaming heads |
| Retrieval score | On needle prompts: how often a head's top-attended token is the needle token being copied (Wu et al., 2024) | Retrieval heads |
| Key outlier ratio | Largest ¦K¦ channel magnitude divided by the median, per KV head | Heads whose keys are hardest to quantize |

Implementation notes:

- A full attention matrix at 32K is 28 heads × 32K × 32K per layer, too large to store. We capture Q and K with forward hooks and compute softmax only for a sample of query positions (for example the last 128), which also matches what decoding does.

- Keys are captured after RoPE, since that is what the KV cache stores and what will be quantized.

- Output: one layer × head heatmap per statistic per task.

# 6. Analysis 2: Which KV Heads Are Sensitive

We quantize one KV head at a time and measure how much the model's output changes. The result is a ranking of all 112 KV heads for each task.

Method:

- Register a custom attention function in transformers that wraps SDPA and fake-quantizes (quantize then dequantize) the post-RoPE K and V of one chosen KV head. Doing this inside attention means prefill also sees the quantized values; a normal single forward pass would otherwise run all attention at FP16 and never test the quantized cache.

- Use 2-bit for the ranking sweep, since 4-bit errors may be too small to separate heads. Re-check the top-ranked heads at 4-bit.

- Score each head by the KL divergence between the FP16 model's next-token distribution and the perturbed one, averaged over tokens. KL needs no labels and is more sensitive than loss. For needle prompts we also record the retrieval-accuracy drop. For GSM8K we use KL on teacher-forced reference solutions, because generating per head is too slow.

- Repeat for each task: 112 heads × 3 tasks, with about 16 samples per task. This is a few thousand forward passes, roughly 1–2 GPU-hours on this GPU (estimate, to be confirmed with a timing run).

Analysis of the rankings:

- Task dependence: Spearman rank correlation between the rankings of each pair of tasks. Low correlation is direct evidence that sensitivity depends on the task.

- Activation vs. sensitivity: correlation between Section 5 statistics (retrieval score, key outlier ratio) and sensitivity. A head that is active is not necessarily sensitive, and this tests it.

- Combined ranking: normalize each task's scores and take the maximum per head, so a head that matters for any task is protected.

- Bit allocation: greedily assign bits from {2, 4, 8} to KV heads under an average-bit budget (for example 3 and 4 bits), most sensitive heads first.

- Validation: compare the mixed allocation against uniform quantization at the same average bits on the held-out test sets.

# 7. Baseline Cost: Before and After Quantization

How quantization is implemented decides whether cost goes up or down. We measure uniform KV quantization against the FP16 baseline in these systems:

| System | KV format | How attention reads it | Expected result |
| --- | --- | --- | --- |
| HuggingFace transformers, default cache | FP16 | Directly | Baseline |
| HuggingFace QuantizedCache (quanto backend) | INT4 and INT2 stored | Dequantized back to FP16 before attention | Memory down, latency up |
| vLLM with kv_cache_dtype="fp8" | FP8 | Fused kernel reads FP8 directly | Memory down, latency down at long context |
| LMDeploy TurboMind (if it runs on Blackwell) | INT8 / INT4 (quant_policy 8 / 4) | Fused kernel with in-register dequantization | Memory down, latency down |

Sweep: context length 4K, 8K, 16K, 32K × batch size 1, 4, 8, for each system.

| Metric | How we measure it |
| --- | --- |
| Time to first token (TTFT) | CUDA events around prefill |
| Time per output token (TPOT) | CUDA events per decode step, median over steps |
| Throughput | Generated tokens per second, per batch size |
| Peak GPU memory | torch.cuda.max_memory_allocated plus nvidia-smi for engines that pre-allocate |
| KV cache size | Computed bytes and measured allocation |
| Maximum batch at 32K | Largest batch that fits on 48 GB, FP16 vs. quantized |
| Quality | The three tasks from Section 4 |
| Energy per token (optional) | NVML cumulative energy counter over a long decode loop |

Profiling to identify the bottleneck (Nsight):

- Nsight Systems: share of each decode step spent in attention vs. linear layers, as context grows. This shows where KV reads start to dominate.

- Nsight Compute on the decode attention kernel: DRAM throughput as a percentage of peak (dram__throughput) and bytes read (dram__bytes_read.sum). Near-peak throughput proves the kernel is bandwidth-bound, so reducing bytes is the lever.

- Roofline: place the FP16 and quantized attention kernels on a roofline using our measured bandwidth.

- Naive path: in the quanto path, find the separate dequantize kernels and the extra memory traffic they cause.

# 8. Measurement Protocol

Every number we report follows the same procedure:

- Persistence mode on; clocks locked with nvidia-smi -lgc where allowed.

- 20 warmup iterations discarded, then 100 timed iterations.

- Median reported with interquartile range, never a single run or a bare mean.

- Run-to-run variance reported next to every speedup claim.

- If clocks cannot be locked, or the GPU is shared, we report the variance and state the limitation.

- Correctness first: outputs compared against an FP32 reference with a relative tolerance before any timing is reported.

# 9. Expected Observations and Plan After Evaluation 1

These are hypotheses that the Evaluation 1 measurements will confirm or reject:

| Hypothesis | Evidence that tests it |
| --- | --- |
| Decode is bandwidth-bound and attention's share of each step grows with context | Nsight Systems breakdown; Nsight Compute DRAM throughput near peak |
| A few heads (early layers, retrieval heads) are far more sensitive than the rest | Long-tailed sensitivity distribution; heatmaps |
| Sensitivity rankings differ by task | Low Spearman correlation between tasks |
| Naive quantized cache saves memory but increases latency | quanto vs. FP16 TPOT and memory |
| Fused kernels save both memory and time at long context | vLLM FP8 / LMDeploy vs. FP16 |
| No existing system supports a different bit-width per head | Survey of system options; this is the gap we fill |

Plan after Evaluation 1 (follows the proposal's critical path):

| Week | Work | Milestone |
| --- | --- | --- |
| 6 | FP16 paged split-K decode kernel, validated against SDPA; quantize_pack kernel with INT4/INT8 packing and fused unpacking | Uniform-precision kernel works |
| 7 | Evaluation 2: end-to-end generation with our kernel, comparison with the Section 7 baselines, first quality numbers | Working prototype |
| 8 | Per-head mixed precision using the Section 6 allocation. Dispatch strategies compared. Llama as second model family | Cost of heterogeneity measured |
| 9 | Final evaluation: capacity demo, ablations, report | Complete |

# 10. Open Issues From the Proposal Review

Reviewing the proposal raised points to fix before or during the kernel work:

## Kernel design

- Runtime branching may show no divergence. If each warp handles one KV head, a per-head bit-width branch is the same for the whole warp. The real costs of runtime dispatch are register pressure (registers sized for the FP16 path), load imbalance between blocks and instruction-cache size. We will measure occupancy, registers per thread and tail idle time, and use the divergence metric to confirm.

- Launch overhead of templated kernels can be removed with CUDA Graphs. We add a third strategy: one launch where each thread block branches on blockIdx (a grouped launch).

- Load balancing: split-K divides work by token count, but INT4 heads read a quarter of the bytes of FP16 heads. We will split by bytes so blocks finish together.

- Page layout for heads with different byte sizes (separate pools per bit-width, or per-head page tables) must be decided.

## Quantization scheme

- The proposal says both "per-token" and "one scale per page". We must pick one.

- KIVI shows keys need per-channel quantization (outlier channels) while values can be per-token. This matters even more for Qwen.

- New tokens during decode need an FP16 buffer until a group is full (as KIVI does).

- Scale and zero-point add about 6% at INT4 (about 4.25 effective bits); this goes into the "average bits" axis.

## Evaluation

- Decode attention's work per byte does not change with batch size, so the "crossover" should be stated over context length, not batch size. There may be no crossover at all, and that is a valid result.

- Energy: use the NVML cumulative energy counter, not instantaneous power samples, which are too coarse for short kernels.

- Perplexity at 32K: compute the loss in chunks; full-vocabulary logits for 32K tokens do not fit in memory.

## Proposal document

- Add a Risks / Fallback section (required by the guidelines), including a hardware fallback.

- Fill in the team split (Section 8.1 is empty).

- Fix the statement that profiling is 20% of the grade (Evaluation 1 is 20% and covers more than profiling).

- Cite which papers claim mixed precision is free, and soften "first real measurement".

- Remove the repeated tables in Section 7; update hardware from the 24 GB target to the RTX PRO 5000.

# 11. Deliverables and Task Split

What we present at Evaluation 1:

- Platform set up and verified (Section 2 checks), with measured peak bandwidth.

- Attention-statistics heatmaps per task (Section 5).

- Per-KV-head sensitivity ranking per task, rank correlations and a proposed bit allocation (Section 6).

- Before/after cost table and plots: TTFT, TPOT, throughput, memory, max batch (Section 7).

- Nsight evidence that decode attention is bandwidth-bound, with a roofline plot.

- Quality numbers for FP16 and uniform quantization on the three tasks.

- Plan of action for weeks 6–9, based on these observations (Section 9).

Workstreams (owners to be agreed by the team; each is independently assessable):

| Workstream | Scope | Owner |
| --- | --- | --- |
| Head analysis | Attention statistics hooks, per-KV-head sensitivity sweep, rankings and bit allocation (Sections 5–6) |  |
| Systems baseline | Cost benchmarks across HF / quanto / vLLM / LMDeploy, Nsight profiling, roofline (Section 7) |  |
| Tasks and quality | Datasets, needle generator, evaluation harness, quality metrics, measurement protocol (Sections 4, 8) |  |